HOUSE BUS

Design Decisions Log

Resolving the gap-analysis priorities, integration-first

Prepared for TJ - Divergent Futures

Version 0.1 - June 2026

# Contents

# Approach

These are the design resolutions for the priority gaps, captured as decided. The through-line is integration: most gaps are closed not by adding a standalone box but by folding the function into the cooling / Octovalve loop that the bus already carries. The unresolved pieces are flagged as Open and deferred deliberately; the logistics get worked out later.

# 1. Ventilation - built into the cooling loop

Decision: There is no standalone ERV. The large HVAC / compressor unit is the ventilation system. It is one machine doing cabin cooling and heating, battery thermal management, Octovalve heat routing, and air ventilation together.

- Filtered fresh-air intake. Outside air is drawn in and filtered (HEPA-class) before entering the cabin - both while driving and when parked.

- Ducted down the side walls. Fresh air is routed down the side walls inside the insulation so the rear/bedroom gets fresh air too, not just the front.

- One integrated unit. Treating cooling, heating, battery thermal, and ventilation as a single system is the whole point - it is why the HVAC has to be sized for all of those jobs at once.

Open: Final duct routing, and confirming the fresh-air rate meets CO2 / occupancy targets. HVAC must be sized to also cool/heat the battery and coordinate with the generator - the core integration-sizing problem.

# 2. Dehumidification + range hood

Decision: A dehumidification path pulls warm, moist air from the upper layer of the cabin back toward the front of the bus and extracts the water from it. A proper range hood over the kitchen removes cooking moisture at the source.

Open: Whether the dehumidifier shares the HVAC loop or runs as its own circuit - to be worked out. (Sharing the loop reuses the compressor and recovers the condensed water; a separate circuit is simpler to control.)

# 3. Drinking & cooking water

Decision: A dedicated built-in stainless-steel potable tank, fed from outside through a hose connection so potable water can be pumped in directly. Drinking/cooking water is kept entirely separate from the recirculating wash water.

This cleanly answers the drinking-water gap: the source is an external fill into a clean SS tank, not the condensate (which stays washing-only).

# 4. Reticulating shower + small inline heater

Decision: A small recirculating (reticulating) shower loop, roughly 10-15 gallons, that continuously filters out hair, dirt, and oils while keeping the heat in the water. An inline electric element trims the temperature back up from whatever it has dropped to.

- The water circulates and is filtered in real time, so a shower uses a fraction of the fresh water of a conventional one.

- Because the recovered/recirculated water is already warm (e.g. it dropped to ~35 C), the inline element only does the final lift - so a 3-4 kW element replaces a 10-12 kW one. Smaller, cheaper, more efficient.

# 5. Emergency egress

Decision: Two independent exits: the main entrance at the front, and an emergency hatch release at the rear with a window at the bed. If the front is blocked, you exit out the back.

# 6. Fire suppression

Decision: Fire extinguishers mounted at the front and at the rear, so one is always reachable from either end of the cabin.

# 7. Glazing

Decision: Double-layered polycarbonate rather than glass: lighter, far more impact-resistant (does not shatter easily), and cheaper. An insulated drop-down cover concealed within the wall can be deployed for an extra insulating layer in very cold conditions.

# 8. Rainwater catchment (optional, designed-in path)

Decision: No committed system yet, but a clean path is reserved. The roof already needs a drainage system; runoff goes to the sides and down as on a normal vehicle roof, to a collection point. A diverter at that point can route rainwater into a holding tank, then through the filters and UV, topping up supply when it is raining.

Open: Whether to include it in V1, and the diverter/holding-tank sizing.

# 9. Mass & payload strategy

Decision: Baseline against a typical 25 ft bus at typical RV weight, then target stripping 20-25% of that mass through advanced materials and smart engineering - carbon-fiber and aluminum replacing brute-force steel where it is doing structural work it does not need to.

Open: Turn this into a running kg/CG budget: list each subsystem's mass, apply the material substitutions, and track against a GVWR/licence target. The 20-25% is the goal to quantify.

# Open items rolled up

- HVAC sizing to simultaneously serve cabin, battery, and the generator interplay (the central integration-sizing problem).

- Dehumidifier: shared HVAC loop vs dedicated circuit.

- Rainwater catchment: include in V1 or not; sizing.

- Mass budget: quantify the 20-25% strip with a real component-by-component tally.

Everything above is captured so it is not lost; the logistics get resolved in the subsystem design passes that follow.